DBS Stepper Control Module			 18-05-2024  (previously 03-03-2024)

Project Overview:

The goal is to drive a Nema17 stepper motor in either direction to a position proportional to a continuously varying 4-24 mA external input or an encoder generated input to a microcontroller. The stepper drives a linear ball screw with a 2mm pitch and “homes” to a limit switch on startup.

Note: The ball screw is “factory set” to approximately two turns from the limit switch in a clockwise direction. This must not be changed however it can change if an error occurs such as an OVERCURRENT EVENT see Error Handling, E1: OVERCURRENT.

The contract scope is to specify and purchase the hardware (excluding the ball screw and stepper), design the enclosure layout, design and supply PCB, assemble components and to write and test the Firmware. Note: Client will pay for hardware purchased by contractor on sighting invoices from contractor. Any damage to hardware whilst under the control of the contractor will be the responsibility of the contractor and where necessary will be replaced by the contractor at the contractor’s expense.

The client retains all I.P. exclusively to project design and custom software.

NOTE: E&OE applies. It is important to note that this briefing document is not prescriptive. It is intended to describe the project only. There will be errors and exclusions in the suggested variables, messages and code flow. It is the contractor’s responsibility to understand the client’s intent and to consult and clarify issues as they arise.

The hardware (selected and purchased by the contractor) includes:

- 1.8 deg, bipolar, Nema17 stepper (low cost for testing purposes)

- Compact stepper driver to suit existing stepper ( Model 17HS19-2004S1-H……Nema 17 High Temp Stepper Motor 59Ncm. Insulation Class H 180C. Data sheet is available). The driver must be able to driver the stepper at the stepper’s maximum rated current. Primary power supply is 12v dc.

Note: the ESP32 firmware… 1/. controls/sets the driver current limits 2/.  monitors the driver current to detect an E1 OVERCURRENT EVENT and 3/. Enables/disables the driver.

- Best microcontroller for the application. Esp32 is preferred but not mandatory.

- Lambda Shield O1M21VOO to interface O2 sensor, including connectors and cables (1813139-1 and 5557). Refer https://controlduino.com/lambda-shield/

- Small industrial quality ENCODER (used as input in “Test” mode).

- Conditioning circuit to interface ENCODER to selected microcontroller.

- Conditioning circuit to interface Hall Sensor for RPM calculation (check if necessary with O1M21VOO).

- 4-24mA input conditioning circuit to interface with the selected microcontroller.

- 5 x 5v(?) LED indicators (O2 sensor-amber, Mode-amber & green and Status- red & green)

- 6th “MAINS” x 12v LED (red)

- Error buzzer

- Power switch for 12v dc supply.

- Pushbutton for reset or toggles txt messages.

- Small Kill relay (on PCB) to drive larger external relay.

- 20 x 4 Backlit LCD screen to display various controller I/O or txt messages.

- 2 position (slide) Mode switch (to select mA input in “Run” mode OR ENCODER input in “Test” mode).

- Home microswitch/limit switch (the stepper drives a linear screw with a 2mm pitch)

- Custom PCB to accommodate esp32, driver, Lambda shield, relay, buzzer, terminals to connect hardware to the enclosure (encoder, switches, p/b, LCD etc. etc.) AND conditioning circuitry for 4-24mA and encode inputs and Hall sensor (H-GTSEN3) for RPM input (check if necessary). NOTE: Switched 12v DC supply goes to PCB.

- Compact metal enclosure to accommodate Master Power switch (supply is 12vDC), PCB, LCD screen, encoder, mode switch, pushbutton, 6 x LED indicators, error buzzer aperture, socket/s for external inputs, socket for stepper connection (including stepper home limit switch) and connector for Comms serial for Arduino IDE. NOTE: the O2 shield, O1M21VOO is also to be incorporated onto the PCB.

- Connector/s for external wiring loom (see schematic).

- Rubber grommets for O2 sensor and 12v supply (see schematic).

NOTE: The Lambda shield plays no part in controlling the stepper other than delaying the ESP32 to control (disable) the stepper for 20 seconds (n seconds) while the Lambda shield heats up the O2 sensor. Other than this, the purpose of the Lambda shield is to display Lambda value, AFR, RPM and Lambda shield error codes.  The stepper position is controlled by either the 4-24ma external input OR the enclosure mounted ENCODER input.

NOTE: The following hardware list is suggested by the client. It is the contractor’s responsibility to check and ensure that the hardware will meet the client’s objectives. The contractor can offer alternatives based on specification and/or value (price).

  - Microcontroller…https://www.adafruit.com/product/5364

  - Stepper Driver…..Amis-30543 Stepper Motor Driver Carrier + HEAT SINK

  - DAC….https://www.dfrobot.com/product-2625.html

  - Pulse generator (encoder)….. similar to RS 475-6173

  - Lambda Shield…. Controlduino O1M21VOO (with Hall Sensor RPM) NOTE: contractor must check that RPM signal from Hall Sensor is compatible.

  - Buzzer… similar to Mouser CMT-1295-85T

  - P/B……  similar to RS 734-682

  - LCD screen…..https://www.amazon.in/Easy-Electronics-20x4-Display-Backlight/dp/B0747NCWSF

- Relay……. Similar to RS 247-4253 (Panasonic AQY212GHAX)

- All LEDs…….3mm diameter

- Mode Switch…..https://www. https://www.amazon.in/Easy-Electronics-20x4-Display-Backlight/dp/B0747NCWSFamazon.in/INVENTO-Position-Switch-Motorcycle-Electrical/dp/B07T1F6JT8/ref=sr_1_2?crid=21EQL5EM50KPT&dib=eyJ2IjoiMSJ9.I6dK3bd5JTSZP-DB6_ER46GnEzDvk6ZH_CdaT7Y0DrR_sp4qEoesoWPGxhzCDEGNb91d12fJDYTpqUWysAI3GmghcnSirH1twf_4peeKCX1S1eX3LRcTMWhjEoEyYeiDC8Rfo7bN0dmMjijg9a2otwghVSOmA0Z8DciJIvn6wYKaQf9XleAfW9-90UJqHxtY4eldXZlKj1NWXFAyNI3-qEv9msPe0IhC7ias9DCP8fDdga9lDQLGut4FRYFpjQsh3biRxbhsI6FJyrsIRG494kbtrc-2M3CT3tJtHju-lh0.SgxZUdGoBpXURDtqFlAQBahYtfOU4wMh1ooKxRaDtjw&dib_tag=se&keywords=2+position+switch+slide&qid=1708750986&s=industrial&sprefix=2+position+switch+slide%2Cindustrial%2C214&sr=1-2

- Power Switch…… similar to mode switch

- Enclosure………similar to Mouser 474-PRT-13838 (exact size to be determined by contractor and approved by client)

- Microswitch….. https://www.amazon.in/Micro-Limit-Switch-Roller-Control/dp/B09C8X9C7Z/ref=sr_1_1?dib=eyJ2IjoiMSJ9.Q9ZUmgba1oqPpV04zRdHtVFtNY3M97hKK_CAZmlMwF4nUhIPXsmDMI9PdXNXO13xuvrv7xfxkQ2TLXhO4foFaykRMAU_OH2JqTBaAs3QMYKn1OJsZwIF7v26RB03M-QqBAxmKBlmy9WAuWd7SzJoXq_Pz5J2CG_qPiorsFmodPmXP7UN3h09Bq-bbemIVsJ2-OaiKnjoDFQ355pKH8jhlt_7fV0VXeD8UnMzDZw6-sG8wHRkO_av_oojmm5

- Connector/s for external wiring loom (see schematic).

- Rubber grommets for O2 sensor and 12v supply (see schematic).

Note: The Amis-30543 Stepper Motor Driver can supply up to 3A and it is recommended that a heat sink is used.  The Model 17HS19-2004S1-H stepper has a rated max current of 2A and in this application I expect the current will not exceed 1A. Although a heat sink may not be necessary, I would like to incorporate it into the design.

Programming Requirements:

- Microcontroller can be esp32 using Arduino IDE. The contractor can suggest another platform for improved performance provided the contractor lists the reasons for such a recommendation and approval is given. Consider given to latency, memory GPIOs etc.

- Contractor must declare whether Event-Driven Code can be or cannot be used and the reasons.

- Code must be written using best practice and all variables must be commented and declared using descriptive names.

- Free access to code for firmware updates and changing variables must be open to client via Arduino IDE (or similar).

- Microcontroller Inputs are:

  - Outputs from Lambda shield (Number required?????)

  - Mode selection switch (“Run” or “Test”)

  - 4-24mA (varying) in “Run” mode

  - ENCODER in “Test” mode

  - rpm pulse up to 1kHz unless O1M21VOO is used (see Hall Sensor Data Sheet)

  - Home limit switch

  - SPI comms from driver (to enable/disable driver and detect overcurrent events etc etc)

  - Reset/toggle pushbutton

  - Spares for future development (please advise what inputs are free)

- Microcontroller Outputs are:

  - Outputs from Lambda shield (ESP32 controls ALL Lambda Shield Data to the LCD display).

  - Enable/disable stepper driver power (controls supply to driver)

  - Stepper driver control inputs

  - Enable/disable small internal relay used to control larger external “kill” relay (kills fuel supply etc etc.)

  - LCD screen inputs (20 x4 Backlit)

  - Comms serial output to Arduino IDE (or similar for Firmware updates etc)

  - Red LED “power” light (unless incorporated into 12v power switch)

  - Amber LED “O2 status” light

  - Red LED “status” light

  - Green LED “status” light

  - Green LED “Run” mode indicator

  - Amber LED “Test” mode indicator

  - Error buzzer

  - Spares for future development (please advise what outputs are free)

- Variables:

NOTE: the variables below ARE NOT PRESCRIPTIVE. They are suggestions to be indicative of the functionality of the code and some of (but not all) the information to be available as LCD display. The contractor will use his/her own variables using descriptive names.

  - V1 = number of steps/rev (360/1.8 = 200 in this case)

  - V2.1 = fast anticlockwise rotation speed while approaching home limit switch (l/s) on power-up. (steps/sec)

  - V2.2 = records the position at state change of l/s on 1st approach. Set = 0.

  - V2.3 = slow clockwise rotation speed while withdrawing from home l/s position (V2.2) until state is changed (steps/sec)

  - V2.4 = position at state change of l/s as the number of steps from V2.2

  - V2.5 = 0 representing “home”

  - V3 = current number of clockwise steps from “home” position (V2.5), representing 0v input

  - V4 = number of clockwise steps from “home” position (V2.5) representing V14(max)

  - V5 = monitors the current position from “home” (V2.5)

  - V6 = rotation speed (steps/sec) during normal operation ie rotation speed

between positions V3 and V4.

  - V7 = oxygen sensor output voltage (Note: v7.1 and v7.2 allow for an alternative sensor with a different range to be used)

  - V7.1 = oxygen sensor minimum value = 0.1v (in this case)

  - V7.2 = oxygen sensor maximum value = 1.0v (in this case)

  - V7.3 = V7 converted to AFR (from Lambda shield)

  - V7.4 = V7 converted to Lambda value (from Lambda shield)

  - V8 = pulse from Hall sensor H-GTSEN3 converted to rpm

  - V8.1 = number of pulses per revolution (initially set to 4)

  - V9 = reset pushbutton state (0/1)

  - V10 = kill relay state (0 = OPEN, 1 = CLOSED)

  - V11 = driver power (0/1)

  - V12 = mode selection switch state (“Run” = 1, “Test” = 0)

  - V13 = ENCODER value converted to volts

  - V13(min) = V14(min)

  - V13(max) = V14(max)

  - V13(init) = V14

  - V14 = controller input voltage (converted from 4-24mA)

  - V14(min) = 0v

  - V14(max) = 3.3v/Esp32, 5v/Arduino?

  - V15 = monitors the stepper driver current

  - V16 = maximum allowable driver current

  - V17 = overcurrent event (0/1)

  - V18 = out of range event (0/1)

Runtime Overview:

The controller can operate in two modes, either “Run” (green LED) or “Test” (amber LED). Run mode uses a voltage input appropriate for the selected microcontroller which is derived from 4- 24 mA external input. “Test” mode uses a voltage input (as for “Run” mode) but derived from the ENCODER ROTATION. In both cases, as either input to the controller varies, the stepper will rotate proportionally within its full normal working range from V3 to V4. A minimum input voltage drives to position V3 and a maximum input voltage drives to position V4. An input of (Vmax + Vmin)/2 will drive to (V3 + V4)/2 etc etc.

NOTE: The ENCODER voltage range is always from V14min to V14max (0-3.3v/Esp32 or 0-5v/Arduino).

The ENCODER takes its initial value (V13init) from the last known value of V14. The ENCODER varies this up or down on rotation of the encoder within the range V14min to V14max and stores this (current) value in V13.

On power up……….. (V8 = 0)

1.  Lambda shield is powered to heat up the O2 sensor. Amber O2 LED is FLASHING.     Programme is delayed 20 seconds.

2. After 20 seconds, Amber O2 LED is ON. The status of V17 and V18 are checked. If V17 OR V18 = 1 there was an overcurrent event (E1) or out of range event (E2) and the programme cannot continue until the RESET procedure is completed. See RESET under ERROR HANDLING, section 3.

If V17 & V18 = 0 (no E1 overcurrent event and no E2 out of range event), the programme can run as follows…

the status of the mode switch is checked (V12 = 1 for “Run”/green mode LED or 0 for “Test”/amber mode LED). In either mode, the “Kill” relay remains OPEN (V10 = 0), the Red status LED is ON, the Green status LED is OFF and Inputs V13 and V14 are monitored but IGNORED. Driver power is enabled (V11 = 1).

- At this point the stepper does a HOMING routing as follows…..

The stepper drives anticlockwise to the “Home” microswitch at speed V2.1 until the l/s changes state. V2.2 is set to 0 and the screw reverses to rotate clockwise at speed V2.3 until the l/s changes state again. The step count from V2.2 is stored in V2.4 and represents the switch hysteresis and any inertial overrun of the first approach. V2.5 is set to zero (V2.5 = 0) and represents “home” at this position.

Any rotation from the “home” position V2.5 = 0 is updated to V5 continuously.

NOTE:  Notice that during the “Homing” move the limit switch changes state twice. The reason is that the first approach is at a higher speed of V2.1 and there is a possibility of inertial overshoot. The stepper reverses at the slower speed V2.3 until the l/s changes state again so as to negate any overshoot.

NOTE: the stepper cannot rotate anticlockwise from “home” (V2.5 = 0) and therefore V5 cannot/should not be negative.

At this point, the programme behaves differently depending on the Mode selection.

4.    If Mode is “Run” on POWER UP,

The stepper responds to input V14, meaning that the stepper will rotate clockwise at speed V6 from “Home” as defined by V2.5 to a position between V3 and V4 depending on the input voltage V14.

If V5 >= V3 OR V5 <= V4 ( i.e. within range, otherwise it is an E2:PAST V3/V4 LIMITS error)

At this point…..

- V12 =1 (Mode = “Run”)

- Red status LED is OFF

- Green status LED is ON

- Green mode LED is ON

- Amber mode LED is OFF

- “kill” relay is CLOSED (V10 = 1).

Display (RUN):

Line1 = Msg12 			  ie. Mode = RUN

Line2 = Msg17 + “ = ” + value(V14)	  ie. MICROSQUIRT INPUT = value(V14)

Line3 = Msg15 + “ = ” + value(V8)	  ie. RPM =  + value(V8)

Line 4 = Msg16 + “ = “ + value(V7.3)	  ie A/F = + value(V7.3)

The above message remains whilst in “Run” mode (with updating variables).

- If the Mode switch is set to “Test” on POWER UP,

V13 is set to V13min (= V14min) and Inputs V13 and V14 are IGNORED as inputs (but still continuously monitored).

- V12 =0

- “kill” relay remains OPEN (V10 = 0).

- Red status LED flashes

- Green status LED is OFF

- Amber mode LED is FLASHING

- Green mode LED is OFF

- Buzzer pulses (beep, beep continuously)

IF V13 = V13min THEN (the encoder has not been rotated)……

Display: Line1 = Msg18 +” = ”+ value(V13)	ie   ENCODER INPUT = value(V13)

Line2 = Msg22 			ie.  SET THE TARGET VALUE

Line3 = Msg23 			ie.  ROTATE KNOB

Line4 = “TBA”

ELSE (V13 > V13min), (the encoder has been rotated)……

Display: Line1 = Msg20 			ie. TO ENABLE TEST MODE

Line2: = Msg5 				ie. PRESS BUTTON TWICE

Line3 = Msg18 +” = ”+ value(V13)       ie. ENCODER INPUT = value(V13)

Line4 = “TBA”

END IF

After user presses button twice (within time limit ?? to avoid accidental touch)…..

  - Kill is still OPEN (V10 = 0)

  - Buzzer is OFF

  - Red status LED is OFF

  - Green status LED is ON

the stepper responds to input V13, meaning that the stepper will rotate clockwise at speed V6 from “Home” as defined by V2.5 to a position between V3 and V4 depending on the input voltage V13 (= V13min-V13max) derived from the ENCODER.

If V5 >= V3 AND V5 <= V4 (i.e. within range, otherwise it is an E2:PAST V3/V4 LIMITS error)

  - set kill CLOSED (V10 = 1)

  - set Amber mode LED is ON (to represent “Test” mode)

  - Green mode LED is OFF

Display (TEST):

Line1 = Msg13 			  ie. Mode = TEST

Line2 = Msg18 + “ = ”+ value(V13)      ie ENCODER INPUT = value(V13)

Line3 = Msg15 + “ = ” + V8 		  ie RPM = value(V8)

Line 4 = Msg16 + “ = “ + V7.3 	  ie. A/F = value(V7.3)

The above message remains whilst in “Test” mode (with updating variables).

NOTE:  If the Mode switch is changed from “Run” to “Test” while V8 > 0 (rpm >0)……..

Since the programme was running in “Run” mode without error, V14 must be valid, therefore the initial ENCODER value can be set to the last “Run” input value V14, meaning that the stepper will not move from it’s last “Run” position when switched from Run to Test while V8 > 0.

Now, V13 is set to V13init and because turning the ENCODER will vary V13, the stepper will rotate to a new position.

  - Set V13init = V14

  - Set V13 = V13init

  - Set V12 =0 (“Test” mode)

  - Kill relay remains CLOSED (V10 = 1)

  - Red status LED remains OFF

  - Green status LED remains ON

  - Green mode LED is OFF

  - set Amber mode LED to ON (to represent “Test” mode)

Display (TEST):

Line1 = Msg13 			 ie. Mode = TEST

Line2 = Msg18 + “ = ”+ value(V13)	 ie. ENCODER INPUT = value(V13)

Line3 = Msg15 + “ = ” + value(V8)	 ie. RPM = value(V8)

Line 4 = Msg16 + “ = “ + value(V7.3)  ie. A/F = value(V7.3)

The above message remains whilst in “Test” mode (with updating variables).

NOTE:  If the Mode switch is changed from “Test” to “Run” while V8 > 0 (rpm >0), the mode changes to “Run” and the controller will respond to input V14, meaning that the stepper will rotate at speed V6 to a position between V3 and V4 corresponding to the input voltage V14.

- V12 =1

- Red status LED is OFF

- Green status LED is ON

- Amber “Test” mode LED is OFF

- Green “Run” mode LED is ON

Display (RUN):

Line1 = Msg12 			  ie. Mode = RUN

Line2 = Msg17 + “ = ” + value(V14)	  ie. MICROSQUIRT INPUT = value(V14)

Line3 = Msg15 + “ = ” + value(V8)	  ie. RPM =  + value(V8)

Line 4 = Msg16 + “ = “ + value(V7.3)	  ie A/F = + value(V7.3)

The above message remains whilst in “Run” mode (with updating variables).

NOTE: 	“Normal Operation” is defined as stepper rotation between V3 and V4 as input voltage varies, irrespective of the Mode (“Run” or “Test”).

NOTE:  During “Normal Operation”, the following variables are continuously updated…………

V3,V4,V5,V7,V7.3,V8,V11,V13,V14,V15,V16.

NOTE:  Re: point 1 above, On Power UP……

When cold, an IC engine produces a lot of moisture in the exhaust system. This can damage the O2 sensor. The solution is to allow the sensor to heat up to operating temperature before the engine starts. This is controlled by delaying the programme flow for 20 seconds before responding to the 4-24mA OR ENCODER input and therefore leaving the “kill” relay open for at least 20 seconds (n seconds). The Lambda Shield controls the sensor heater and during this “heating” delay the amber O2 LED is FLASHING. After 20 seconds, the amber O2 LED is ON.

Error Handling:

1./   E1: OVERCURRENT

Driver current is continuously monitored and stored in V15 and the maximum allowable current is set by V16. An “E1: OVERCURRENT ” event occurs if V15 exceeds V16. Two most likely causes are A: screw nut passes home microswitch (driving anticlockwise) and hits a hard stop OR B: screw nut passes position V4 (driving clockwise) and hits a hard stop. Other reasons (not specified), if they occur are handled in the same way. See Diagnosing Problems (not included here).

If an E1: OVERCURRENT EVENT occurs…..

- V17 = 1

- Driver current is OFF (V11 = 0)

- “KILL” relay is open (V10 = 0)

- Buzzer pulses

- Red status LED flashing

- Green status LED is OFF

E1 Display 1:

Line 1…. Msg1	                       ie. CRITICAL FAULT

Line 2… Msg2		          ie. E1:OVERCURRENT

Line 3….Msg11		          ie. MANUAL RESET REQD

Line 4….Msg6		          ie. DRIVER POWER IS OFF

E1 Display 2:

Line 1… Msg2		          ie. E1:OVERCURRENT

Line 2….Msg11		          ie. MANUAL RESET REQD

Line 3….Msg8  		          ie. TURN MASTER PWR OFF

Line 4…. Msg7 		          ie  CHECK BALLSCREW

NOTE:  Display 1 and Display 2 alternate on for 2 seconds each and repeat indefinitely until Master Power is OFF.

Note: At this point, user should have identified the cause of the E1: OVERCURRENT EVENT and fixed it. Refer Diagnosing Problems (not included here).  The ball screw should be rotated manually in a clockwise direction so that it passes the home limit switch by approximately two turns (as per the “factory” setting).

2./    E2:PAST V3/V4 LIMITS

During Normal Operation the screw nut position moves between V3 and V4 in response to the input voltage (irrespective of the Mode). The position of the screw nut wrt V2.5 (“home” position) is stored in V5.

An E2:PAST V3/V4 LIMITS error occurs if V5 < V3 or V5 > V4.

Note: If V5<V3, the AFR may be excessively high (check). If V5>V4, the AFR will be excessively low (check). If V5 < 0 (V2.5 = 0…”home”), the screw nut has passed the l/s and may be on it’s way to a hard stop. See Diagnosing Problems (not included here).

- V18 =1

- Driver current is OFF	(V11 = 0)

- “KILL” relay is open	(V10 = 0)

- Buzzer pulses

- Red status LED flashing

E2 Display 1:

Line 1…. Msg1	                       ie. CRITICAL FAULT

Line 2… Msg 3  		          ie. E2:PAST V3/V4 LIMITS

Line 3….Msg11		          ie. MANUAL RESET REQD

Line 4….Msg6		          ie. DRIVER POWER IS OFF

E2 Display 2:

Line 1… Msg1		          ie. CRITICAL FAULT

Line 2….Msg11		          ie. MANUAL RESET REQD

Line 3….Msg8  		          ie. TURN MASTER PWR OFF

Line 4….Msg10 		          ie  CHK VALUES V3/V4/V5

NOTE:  Display 1 and Display 2 alternate on for 2 seconds each and repeat indefinitely until Master Power is OFF.

Note: At this point, user should have identified the cause of the E2:PAST V3/V4 LIMITS EVENT and fixed it. Refer Diagnosing Problems (not included here). The ball screw should be rotated manually in a clockwise direction so that it passes the home limit switch by approximately two turns (as per the “factory” setting).

3/. RESET (after E1 or E2 error event)

NOTE: it is assumed that the user switched off the power and fixed the cause of the ERROR event.

As V17 =1 and/or V18 =1, the programme cannot be restarted until the following RESET procedure is completed in the following order….

- Master Power ON

- Driver current is OFF (V11 = 0)

- “KILL” relay is open    (V10 = 0)

- Buzzer pulses

- Red status LED flashing

- Green status LED is OFF

RESET Display 1:

Line1….Msg1			ie.  CRITICAL FAULT

Line2….Msg25			ie.  FAULT: E1 and/or E2

Line3….Msg11  			ie.. MANUAL RESET REQD

Line4….Msg5  			ie.  PRESS BUTTON TWICE

USER presses the button twice (within 2seconds?)…..

- V17 = 0 and V18 = 0

- Driver current is ON  (V11 = 1)

- “KILL” relay is closed  (V10 = 1)

- Buzzer STOPS

- Red status LED is OFF

- Green status LED is ON

RESET Display 2:

Line 1:…Msg9 				ie. NORMAL OPERATION

Line 2….Msg26			ie. SELECT MODE RUN/TEST

At this point the programme runs as normal in either “Run” or “Test” mode because V17 and V18 = 0

Note:  if the cause of the E1 and/or E2 error events were not fixed it/they will occur again etc etc.

Messaging:

At all times the following variables are continuously updated: V7, V8, V13, V14 and V15.     V5 is the position with respect to the “home” (after V2.5 is set =0) and is continuously updated during NORMAL OPERATION.

NOTE: During NORMAL OPERATION (in either “Run” or “Test” Mode), the displayed messages are either Display (RUN) OR Display (TEST). These messages stay displayed indefinitely until the user presses the P/B.

Pushing the P/B in either MODE will interrupt the RUN or TEST Displays and will cycle through a list of Messages. The list will include 1/ some of those listed below 2/ some of the variable values and 3/ data that can be pulled from the Lambda shield and driver via SPI.

The contractor will code for this and set it up with one or two messages as examples from each type 1,2 &3 mentioned above. The complete list will be compiled by the client.

The RUN or Test messages will redisplay if the P/B is not pressed for 5 seconds.

Msg1: “CRITICAL FAULT”

Msg2: “E1: OVERCURRENT”

Msg3: “E2:PAST V3/V4 LIMITS”

Msg4: “RESET”

Msg5: “PRESS BUTTON TWICE”

Msg6: “DRIVER POWER IS OFF”

Msg7: “CHECK BALLSCREW”

Msg8: “TURN MASTER PWR OFF”

Msg9: “NORMAL OPERATION”

Msg10: “CHK VALUES V3/V4/V5”

Msg11: “MANUAL RESET REQD”

Msg12: “Mode = RUN”

Msg13: “Mode = TEST”

Msg14: “PWR OFF 4 TEST MODE”

Msg15: “RPM = ” + value(V8)

Msg16: “A/F = ” + value(V7)

Msg17: “MICROSQUIRT INPUT”

Msg18: “ENCODER INPUT

Msg19: “SET ENCODER TO = 0V”

Msg20: “TO ENABLE TEST MODE”

Msg21: “TO STAY IN RUN MODE”

Msg22: “SET THE TARGET VALUE”

Msg23: “ROTATE KNOB”

Msg24: “V3/V5/V4 = ”

Msg25: “FAULT: E1 and/or E2”

Msg26: “SELECT MODE RUN/TEST”

Serial Output:

The serial port is to interface with the Arduino IDE (or similar for PC). Firmware updates and changes to variables must be accessible by the client.

ACKNOWLEDGEMENT and ACCEPTANCE

By signing this document, the contractor acknowledges that he/she has read the document carefully and undertakes to fulfill the contract for the agreed price and terms. The contractor also acknowledges that this document is not prescriptive and undertakes to consult with the client where there are uncertainties and/or errors in relation to the client’s intent and where a better approach/method or hardware could be used (in the opinion of the contractor).

SIGNED:

CONTRACTOR NAME: